# How to Upgrade Camera Operation System VCRT

Please do the following steps  in order to update the

VCRT Version of your camera.

1) use the shell command 'er'

2) download VCRT??.msf ('lo')

3) reset camera

4) download vs?.msf ('lo')

5) start vs and select item 5

Please select the correct VCRT??.msf and vs?.msf version from the table below !

Attention:

Please, no power reset during programming sector 0

The loader to use for an operation system upgrade depends on the OS perviously intalled on the camera!

| Camera | old version | latest version | Use this loader if OS VCRT2.2 or previous versions are already installed | Use this loader if OS VCRT2.3 is installed |
| --- | --- | --- | --- | --- |
| VC11/13/21 | VCRT2.1 (VCRT21g.msf) | VCRT2.33 (VC23311.msf) | VS1.msf | VS3.msf |
| VC51/61 | VCRT2.2 (VCRT51.msf) | VCRT2.31 (VC23151.msf) | VS1.msf | VS3.msf |
| VC65 | VCRT2.2 (VCRT65.msf) | VCRT2.33 (VC23365.msf) | VS1.msf | VS3.msf |
| VC67 | VCRT2.2 (VCRT67.msf) | VCRT2.31 (VC23167.msf) | VS1.msf | VS3.msf |
| VC38 | VCRT2.32 (VC23238.msf) | VCRT2.33 (VC23338.msf) |  | VS4.msf |